Content of the Tech-Check Lists is based on several sources including MASTERSPEC®.

tech-check architectural  I small commercial

This is a template, so some items will apply to your project, and others won’t or will be missing.

When using the list, make the changes needed for your project.

| ## ◻ | ## reference documents - DIMENSIONING -“Dimensioning Guidelines for Architectural Projects”. - OTHER CHECK LISTS – “Building Code” and “Accessibility.” - NOTATION HELP – “Drawing Notation,” “General Notes for Small Commercial.” - Coordinate abbreviations with list in Cover Sheet |
| --- | --- |
| ## ◻ | ## TITLE BLOCK - Project name and Address. - Owner’s legal name and address. (check with Google) - Issue Number, Description (Issue for Bid, Addendum A, etc.) and date of issue. - Additional information such as general contractor, when known and approved by Principal. - Initials in “Drawn By” and “Checked By”; plot date. - Stamp. - Scale, Project Number. - Drawing Title / Drawing Number. - Deltas for Addenda and subsequent Bulletins when applicable. |
| ## ◻ | ## Always do this: - Number SIMILAR components for clear notation: Glass #1, Glass #2; Storefront #1, Storefront #2, etc. Note the designations in the Schedule of Finishes and other schedules and refer to the components by its designation throughout the set. - Check that the designations in the schedules match the designations on the drawings. - Always talk to reps. Confirm capacities, sizes, configuration, availability, use. Send your plans for reviewing when applicable. - If an item is typical and you know it, do not add other labels. Add UON only when you know that there is, or probably is, a different condition or materials from the one documented.n you know that there is, or probably is, a different condition or materials from the one documented. |
| ## ◻ | ## GRAPHICS - Indicate with dashed lines and label overhead components at building exterior: canopies, awnings, and such. Label them, for example, “CANOPY ABOVE”. - Wall and roof openings: Indicate with an “X” and label “OPEN”. - “North” arrow in every floor plan – oriented in the same way. - Graphic bars (these are required by some agencies - check). - Indicate areas in Project Site which are not included in the Contract and label: NIC (“BY OTHERS” can be confusing. Use “NIC” instead.) - Project Site indicated clearly in building plans. |
| ## ◻ | ## Dimensioning - Height dimensions for architectural projects are from top of slab (TOC), or top of sheathing. - Horizontal dimensions for architectural work are to center of framing, face of concrete, face of sheathing, face of gypsum, and such, except critical dimensions (including exiting) which are to “face of finish”. - Order of dimension strings: - First String: Total building dimension (wall-to-wall), closed string. - Second String: Dimensions to structural elements (columns, etc.), closed string. - Next Strings: Components in order of importance; tied to the structure to fix them, open strings. - Critical dimensions (exiting, accommodate equipment, and such) are labeled “CLEAR”. - Generally, do not set dimensioning to less than 1/8”. Note 1/16” only when you know it can be done in the field or it can be manufactured to that precision level. |
| ## ◻ | ## Consistency - Terminology is the same throughout. - Components are tagged with the same tag throughout the set. - Work which is the same has the same tab. - Address only the contractor, no one else, as if he was in front of you: “Do this”, “Do that”. - Graphics representing the same item or work are consistent: Size, type, etc., (for example, finishes, types of partitions, and such). |
| ## ◻ | ## NO-NOs - Do not describe building envelope finishes and other elevation information in the floor plans. - Do not use periods in the abbreviations: (UON not U.O.N). - DO NOT INCLUDE RATED ASSEMBLIES THAT DO NOT HAVE A UL NUMBER (or other number by any other testing agency). NEVER DESIGN YOUR OWN RATED ASSEMBLIES! - DO NOT use the UON all the time, use ONLY WHEN NEEDED. When you use UON you are telling the contractor that somewhere in the documents there is or there might be a condition which is different to the one with the UON tag, so the contractor has to search all the documents for it, so unless this is the case, drop the UON. |

1 GENERAL

2 COVER SHEET and FRONT-END SHEETS

| ## ◻ | ## COORDINATION WITH THE SET - Coordinate abbreviations used in the set with list in Cover Sheet |
| --- | --- |
| ## ◻ | ## include - Abbreviations - General notes – check that they are applicable to the project. - Project name and address. - Title 24 notes. - Drawing List. - Project Directory. - Vicinity Map. - Project Information and code data. - Separate permits and deferred submittals. - Project Legend - Graphic symbols used on the project. |
| ## ◻ | ## REQUIREMENTS - Code Analysis and calculations - Calculations for exiting requirements and for fixture requirements. - CALGreen Checklist - Separate Sheet - ADA/ Accessibility Path of Travel Layouts - Accessibility Standard Details (maneuvering clearances, signage, etc.) - Compliance Statements if required by the local Authority Having Jurisdiction |

| ## ◻ | ## Coordination with consultants: - Building footprint. - Property lines and easements. - Site topography (grading). - Ramps, curb ramps, sidewalks/ walkways, steps or stairs: Locations and configurations. - Parking layout and sloping. - Trees and planting areas. - Utilities. - Exterior lighting. - Signage. - Verify location of new landscape areas; include footage if required for permitting. - Utility lines or service points of connection. |
| --- | --- |
| ## ◻ | ## SITE INFORMATION: ## (WHEN POSSIBLE obtain a list of what should be shown in the site plan from the local jurisdiction.) - Parcel number. - Access to site from point of connection to public road. - Access roads, driveways, turn-arounds, including with and slope. - Property lines: - Distance between buildings and property lines. - Required fire rating in selected portions of the building due to distance from property lines. - Easements, dimensioned, located in plan, and labeled (utilities, right-of-way, etc.) - Setbacks. |
| ## ◻ | ## SITE CONSTRUCTION - Trash enclosure, elevations, plans, and details. Check if the trash enclosures are required to have a roof and drains. (Trash enclosures do not have doors, they have gates). - Bicycle lockers and racks. - Accessibility details: Signage, and tactile surfacing. - Accessible parking stalls plans. - Access ramps and pipe railings. - Location of entrance signage. - Fences and gates located in reference to property lines when applicable. - References to details are coordinated. |
| ## ◻ | ## PARKING: - Driveways, access, exiting. - Pedestrian traffic. - Parking layout, number of slots, dimensions, flow of traffic, signage. - Accessible path of travel. - Parking signage. - Parking appurtenances. - Fire hydrants if required by permitting agency. |

3 SITE PLAN(S) - FOR PROJECTS WHERE SITE WORK IS INCLUDED

| ## ◻ | ## COORDINATION WITH the PROJECT TEAM - Coordinate with electrical engineer for locations of electrical service to architectural components, including interior and exterior lighting, appliances and equipment, etc. - Coordinate dimensions of wall and roof openings with the structural engineer. |
| --- | --- |
| ## ◻ | ## Coordination within the set - The following key references are indicated and correctly labeled: - Building elevations - show arrow perpendicular to elevation (“go around” the building to check) Tag elevations by location (North, South,Southeast) - Building sections - end arrows pointing the right way. - Dimensions: - First strings at width and length of building: Wall to wall, ONE DIMENSION, closed string. - Second strings at with and length of building: Wall-to-wall, column lines or column centers, closed string. - Next Strings: Main components (openings, changes in plan, etc.) open string (one dimension open to allow for Contractor’s adjustment - choose the least critical dimensions) - Dimensions in the elevations and sections are indicated in the same format as in the floor plans. - References to building sections are coordinated. - References to details are coordinated. - Stairs, if any, are indicated and tagged. |
| ## ◻ | ## NO-NOs - Do not DESCRIBE building exterior finishes and other elevation information in the floor plans. |
| ## ◻ | ## GRAPHICS - Indicate overhead components at building exterior (awnings, roof components, etc) with dash-lines and label, for example: “CANOPY ABOVE”. - Project site indicated clearly. - Indicate areas in Project Site which are not included in the Contract and label: NIC. |

4 FLOOR PLANS

| ## ◻ | ## GENERAL Coordination - The following key references are indicated and correctly labeled: - Details. - Enlarged plans. - Type of roofing assembly has been confirmed with the specifier and the roofing manufacturer representative. - Details shown correspond to the applicable roof type. - Roof dimensions are shown. - References to building sections. - References to details. |
| --- | --- |
| ## ◻ | ## Terms - Roof Membrane: Single Ply (can be either TPO or PVC, but do not label until you know for sure) - Board insulation. - Protection board if under roof membrane - Cover board if over roof membrane – check roofing assembly with specifier. |
| ## ◻ | ## ROOF CONSTRUCTION - Heights are to top of sheathing, steel deck, or concrete (not Floor). - The following is indicated: - Parapets. - Perimeter drainage. - High and low points of the roof. - Slope directions and slope ratios. Cricketing. - Roof drains and overflow drains - Mechanical/ Electrical/ Plumbing units (future location - dashed - if not in the contract) - Maintenance pads. - Expansion Joints. - Seismic Joints. |
| ## ◻ | ## roof appurtenances - Walking pads - Roof hatches. - Ladders at roof perimeter. - Skylights. - Perimeter fall protection rails, if any. |

5 ROOF PLAN

| ## ◻ | ## NO-NOs - Do not indicate building envelope finishes and other elevation information in the floor plans. - DO NOT describe materials in the elevation notations, just identify them (GL1, GL2, etc.) UNLESS IT IS NECESSARY FOR CLARITY. Describe materials in the Exterior Finishes Schedule. |
| --- | --- |
| ## ◻ | ## COORDINATION WITH THE DESIGN TEAM - Structural components. - Exterior levels. |
| ◻ | ## DIMENSIONING - Horizontal Dimensions: - First dimension: Exterior wall to exterior wall – match floor plans. - Second dimension: Column lines. - Vertical Dimensions: - Heights: To top of sheathing, top of concrete, top of parapet, top of equipment screen. - Heights of canopies, and other appurtenance. - Openings: To face of opening (framing or concrete) except if critical (dimension only to locate if the item in the opening if the item dimensions are indicated in a schedule, such as doors, windows, etc. |
| ◻ | ## INDICATE: - Gridlines. - If possible, indicate items in the front plane with one line weight and items in the back planes with a lighter line weight. - Materials tagged on all sides of the building by type (for example, GL1, GL2, and included as such in the Exterior Finish Schedule. |
| ◻ | ## GRAPHICS - Sections at least in two directions. - Grid lines. - One section for every type of exterior wall arranged in the sheet in order. - Heights to top of parapet, top of sheathing or top of slab. - Heights to top of openings. - Materials tagged. - References to details coordinated. - Building heights are show to TOP OF SLAB (not floor) |
| ◻ | ## Exterior doors - Tagged correctly in the following categories: Hollow Metal (no “man doors”), overhead coiling doors (motor operated or manually operated), overhead sectional doors, aluminum framed entrance doors, or all-glass entrance doors. - Tags are coordinated with the door schedule. - Elevations of all doors are included in the door schedule. - Hardware groups are included (except for hollow metal doors, typically hardware in other exterior doors is provided by the door manufacturer) |
| ## ◻ | ## aluminum glazed assemblies - Tagged correctly in the following categories: - Storefront: From top of slab to underside of floor deck above not to exceed 14 feet high. - Punched Windows: Single openings in the building envelope. - Glass types identified and tagged. |
|  | EXTERIOR FINISHES SCHEDULE - Materials are grouped by types (all glass types, etc). - Tags in the elevation are the same as in the Exterior Finishes Schedule. These are examples to illustrate nomenclature and listing; products named are examples. ## GLASS - INSULATING - Glass Type: Insulated. - Exterior Panel: 1/2” thick, tinted with Low E coating in #2 Surface. - Interior Panel: 1/2” thick, clear - Interspace: Air, dehydrated - Total Thickness: 1”. - Product: [Solarban 60 by PPG] [Insert] ## ALUMINUM FRAMING: - Product: X by [Kawneer] [Insert]. - Dimensions: 2” by 4-1/2”. - Assembly: Stick. - Finish: [Clear anodized] [[Insert]. ## STUCCO (CEMENT PLASTERING): - Finish Texture: [Insert]. - Color: [Insert]. ## THIN BRICK VENEER: - Type: Clay Face Brick. - Unit Size: X wide by X height by X long. - Product: X by Basalite, colors [insert]. - Installation: [thinset over concrete] [mortar bed] [description] |

6 BUILDING ELEVATIONS

| ## ◻ | ## COORDINATION WITH THE DESIGN TEAM - Structural components. - Exterior levels. |
| --- | --- |
| ## ◻ | - Sections at least in two directions. - Grid lines. - One section for every type of exterior wall arranged in the sheet in order. - Grid lines. - Heights to top of parapet and top of sheathing, building clear height, and top of openings. - Materials tagged. - References to details. |

7 BUILDING SECTIONS

| ## ◻ | ## COORDINATION IN THE SHEET - As much as possible keep details of the same family together. - Check that in details that include the same assembly, the assembly is the same (size of door frames, type of floor slab, etc. - Check that items in the details are tagged in the same manner (overhead deck is always overhead deck”, “window” is always “window”, etc. |
| --- | --- |
| ## ◻ | ## GRAPHICS - Details are grouped by families. - Graphic representation of items is consistent. - Terminology is consistent and correct. |
| ## ◻ | ## ROOF - Parapet showing coping and transition to roof. - Roof drain. - Downspout - Skylight, plan and section at curb. - Scooper at roof, including scupper box and flashing box. - Roof hatches - show post and railings. |
| ## ◻ | ## Building envelope - Enlarged sections showing each type of wall with tags coordinated with other drawings. - Transitions between materials are detailed. - Detail of openings head, jamb, sill as applicable to openings, with coordinated references - Swing of doors shown. - Glass types identified and tagged. - Parapets. - Extent of Stucco - Indicate location of expansion joints at 12 feet on center each way MINIMUM. If not indicated, the Contractor will choose the locations. |
| ## ◻ | ## DOOR STANDARDS - Typical: Door signage (Doors to remain unlock, etc.): Storefront entrance. - Typical: Location of door devices. - Typical: Door clearances. |

8 building details - GENERAL

| ## ◻ | ## DIMENSIONING - Dimensions are to column grid. Critical dimensions are “clear.” - Heights are to top of concrete (for composite decks) or top of steel (for steel decks). |
| --- | --- |
| ## ◻ | ## DOORS - Doors are tagged - only in the floor plans. - Interior door numbers correspond to the room numbers. Exterior door numbers are tagged correctly. - Doors in rooms with multiple doors are tagged sequentially, for example, doors in Room 1001 are tagged as 1001A, 1001B, 1001C, etc. - Door tags are coordinated with the Door & Frame Schedule and the Hardware Groups. - Required clearances at doorways (doors and gates) are correct and labeled. - Direction of door swing meets code. |
| ## ◻ | ## Partitions - Partition types selected do not exceed the maximum height indicated in the partition details. If you have questions on this, call Gloria or Josie - Partitions are tagged one by one or in group (note “PARTITION TYPE A”). - Fire rated partitions are correct for the fire ratings required. - Depths required for recessed items (fire extinguisher cabinets, bathroom accessories, etc.) are coordinated with the partition types assigned. |
| ## ◻ | ## EXITING WIDTHS - Required minimum clear exiting widths at stairs, hallways and corridors are provided or exceeded and labeled as “CLEAR”. |
| ## ◻ | ## Fire extinguishers - Travel distances to nearest fire extinguisher are indicated (MAX 75’). - Cabinet types are labeled: Rated or non-rated, recessed; semi-recessed; surface-mounted. - Partition type widths are sufficient to accommodate recessed and semi-recessed cabinets. - References to drawing and detail number that shows accessible mounting heights for cabinets are correct. |
| ## ◻ | ## SUSPENDED CEILINGS - Coordinate dimensions of HVAC and plumbing with the mechanical engineer. - Coordinate locations of fire protection systems with the design-build engineer. - Schedule: Products including tiles or panels and suspension systems. - Note that all suspended systems are “Heavy Duty”. - References to details are coordinated. - Layout does not include tiles or panels smaller than ½ a unit or are cut in a way they cannot be suspended. - Extent of each new ceiling is shown. - Ceiling and Soffit heights indicated; +9’-0” AFF. - Light fixtures - Layout of ceiling grids. - Ceiling-mounted items, such as air supply diffusers, return air grilles, exit signs, are indicated. |
| ## ◻ | ## FINISHES - Coordinate with the interior designer. |
| ## ◻ | ## INTERIOR STAIRS - Compliance with the CBC: - Widths. - Projections on the width. - Headroom clearances. - Tread size and configuration. - Handrails configuration and heights. - Handrails extensions at top and bottom of stairs. ## CHECK THAT Space under stairs is open. |

9 interiors

| ## ◻ | ## FINISHES SCHEDULE - Materials are grouped by types (same type of products together; paint colors together, etc.) - Tags in the interior elevations and details are the same as in the Finish Schedule. - Correct terminology. Refer to Template attached. - TAGS ARE TAKEN FROM THE SPECS. Change them is you are used to other designations. |
| --- | --- |

10 SCHEDULES

SCHEDULES ORGANIZATION

11 RCP SCHEDULE

CHECK typical info (entries are examples only)

## APC1

- Type:  Acoustical Panel Ceiling

- Classification:  Heavy Duty

- Panel Product: [Insert manufacturer and product]

- Color: White [Insert]

- Size: 24” by 24” [Insert]

- Suspension System Product: [Insert manufacturer and product]

- Suspension System Color: White [Insert]

## GC1

- Type:  Gypsum Suspended Ceiling

- Panel Product: USG SHEETROCK® Ultra-light panels

- Thickness: 1/2”

- Suspension System Typ:  USG Drywall Suspension System

- Suspension System at Corridors: USG Drywall

- Classification:  Heavy Duty

## NOTE: GYPSUM BOARD CEILINGS CAN ALSO BE SUSPENDED WITH REGULAR STEEL FRAMING: STUDS AND CHANNELS.

12 INTERIOR FINISHES SCHEDULE

Floor finishes

## CF1 CONCRETE FLOOR

- Finish: Polished.

- Product: [Insert].

- Color: [Insert].

- Polish Level: [Insert].

- Contact: [Insert rep’s info]

## CF2 CONCRETE FLOOR

- Finish: Sealed

- Product: [Insert Sealer-Densifier]

- Contact: [Insert rep’s info]

- Contact: [Insert rep’s info]

## CS1 CARPET SHEET

- Product: [Insert manufacturer and product].

- Color: [Insert]

- Installation: [Direct-glue-down installation [Double-glue-down installation]

- Contact: [Insert rep’s info]

## CT1 CARPET TILE

- Product: [Insert].

- Color:[Insert]

- Size: [Insert]

- Contact: [Insert rep’s info]

## RF1 RESILIENT FLOORING

- Product: [Insert].

- Material:  Sheet, Vinyl.

- Thickness:  1/8 inch.

- Color: [Insert]

- Contact: [Insert rep’s info]

## RF2 RESILIEN TILE FLOORING

- Product: [Insert Manufacturer and Product]

- Material:  Recycled Rubber [Insert]

- Thickness:  1/8 inch [Insert]

- Size: 36” by 36” [Insert]

- Color: [Insert]

- Contact: [Insert rep’s info]

## CERAMIC TILE FLOORING

- Product: [Insert].

- Material:  Ceramic Tile.

- Size: [Insert]

- Thickness:  1/8 inch.

- Color: [Insert]

- Finish (for stone)

- Contact: [Insert rep’s info]

## WB1 WALL BASE

- Wall Base Type:  [Coved] [Straight}

- Wall Base Product: [Insert]

- Wall Base Color:  [Insert]

- Contact: [Insert rep’s info]

Wall Finishes

## WP1 WALL PAINT

- Color: [Insert]

- Manufacturer: [Insert]

## NOTE: paint MANUFACTURERS IN THE FINISH  SCHEDULES ARE LISTED ONLY FOR COLOR IDENTIFICATION.  THE MANUFACTURES of products to be used in the project are included in the Specifications, scheduled by substrates and listed as systems.

## WP1 FIBER-REINFORCED WALL PANELS (FRP) at Janitor Rooms (utilitarian):

- Product: [Marlite] [Insert].

- Color: [Insert]

## WT1 WALL TILE

- Product: [Insert product and manufacturer]

- Type: [Ceramic tile] [Insert]

- Face Size: [Insert]

- Thickness: [Insert]

- Color: [Insert]

- Installation: [Thin set over gypsum board] [Thiset over waterproofing over tile backer board]

- Grout Product:

- Grout Color:

- Contact: [Insert rep’s info]

Architectural Woodwork

## CW1 CABINET WORK plastic laminate clad

- AWI Grade: Custom, Flush Overlay

- Core Material: MDF

- Exposed Surfaces Material: Plastic Laminate

- Product: [Insert]

- Color: [Insert]

- Semi-Exposed Surfaces Material:  Melamine

- Color: White [Insert]

- Hinges: European Type, 180 degree opening unless opening is restricted by adjacent construction, self-closing

- Shelf Support:  Adjustable, installed at sides

- Wire Pulls:  Back Mounted, 5” long

- Drawer Slides: Side mounted, full extension

- Exposed Hardware Finish:  [Satin Stainless Steel] [Insert]

## CT1 COUNTERTOP

- AWI Grade: Custom

- Core Material: MDF

- Exposed Surfaces Material: Plastic Laminate

- Product: [Insert]

- Color: [Insert]

- Contact: [Insert rep’s info]

- Semi-Exposed Surfaces: Melamine

- Color: [White] [Black]

- Edge: As detailed [Insert]

- Backsplash: As detailed [Insert]

## CT2 COUNTERTOP

- Material: Quartz Agglomerate [1/2”] [3/4”] thick

- Product: [Insert]

- Color: [Insert]

- Contact: [Insert rep’s info]

- Edge: As detailed [Insert]

- Backsplash: [1/2”] [3/4”]

- Backsplash Top: As detailed

- Joints: Bonded.

Glazed Assemblies

## GL1 GLASS for sidelites.

- Type:  Safety Glass - [Fully Tempered Single Panel] [ Laminated]

- Color: [Clear] [Insert]

- Thickness: As required by structural performance but not less than 1/2inch

BATHROOM ACCESSORIES

## entries are examples only; check your project.

## Shower accessories

- Bench for side wall in Dressing/Shower Area, roll-in compartment.

- Product:  Bobrick B5193 Folding Shower/Dressing Area

- Towel Bar wall mounted 24” long

- Product:  Bobrick B 530 x 24 Extra-Heavy Towel Bar

- Surface-Mounted Soap Dish:

- Product;  Bobrick B68

- Curtain Rod 48” long

- Bobrick B6103 x48

- Shower Vinyl Curtain:

- Product: Bobrick V204 curtain with B204 hooks

- Contact: [Insert rep’s info]

13 DOOR SCHEDULE minimum INFORMATION

## CHECK Typical Information (examples only)

- Room Name and Number

- Door Number

- Door Dimensions

- Door Thickness:

- Wood doors:

- Product:

- Door Type: Flush wood

- AWI Grade: Custom

- Door Face: MDF painted.

- Color: [Insert]

- Hollow Metal Doors:

- Product:

- Grade:

- Door Type:  Flush SDI

- Face: Primed steel to be field painted

- Aluminum Frames:

- Product:

- Details at Head, Jamb and Threshold

- Glass Type

- Louver Type

- Assembly Rating (be consistent; indicate either minutes or hours, not both; i.e. 20-MINS, 90-MINS; not 20-MINS, 1 ½ Hrs.)

- Hardware groups and threshold.

- Miscellaneous information, such as Card Reader where applicable, etc.

- Remarks – for example: “OWNER-PROVIDED, CONTRACTOR TO INSTALL”.

DOOR SCHEDULE graphics

- Doors:

- Sizes and types of units, by elevation.

- Details of head, jamb at door and sidelight.

- Hardware mounting heights.

- Security and access systems mounting heights.

- Aluminum or hollow metal frames.

- Vision panels and louvers shown.

- Frames material and finish.

END OF TECH-CHECK LIST