The content of the Tech-Check Lists is based on our specs and MASTERSP

tech-check specs architectural  I Division 06

This is a template, so some items will apply to your project, and others won’t or will be missing.

When using the list, make the changes needed for your project.

EC®.

| ## ◻ | ## 06053 - MISCELLANEOUS ROUGH CARPENTRY |
| --- | --- |
|  | - This section mainly specifies concealed carpentry used for supporting, shimming, or other auxiliary functions. - Miscellaneous carpentry is tagged either as preservative treated (i.e., when in touch with concrete) or fire-retardant treated where required by code (i.e. concealed blocking or nailers). - Wood is either fire-rated or preservative-treated, not both. - Plywood utility panels are tagged as fire-treated and not painted so marks are legible for inspection. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 064111 - PLYWOOD OR BOaRD (MDF) paneling |
|  | - This section specifies wood composite board or plywood paneling, painted or with transparent finish. - Miscellaneous carpentry is tagged either as preservative treated (i.e., when in touch with concrete) or fire-retardant treated where required by code. - Wood is either fire-rated or preservative-treated, not both. - Plywood utility panels are tagged as fire-treated and not painted so marks are legible for inspection. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 064113 - wood-veneer-FACED ARCHITECTURAL CABINETS - see images at the end of this list |
|  | - Grade is AWI Premium. - Construction: Frameless. - Door and Drawer Front: Flush overlay. - Exposed Surfaces: Wood Veneer - Include the following in the specs or the drawings: - If in the Project there is paneling with the same veneer, tab the cabinetwork veneer as “Match Paneling.” - Otherwise, the following note in the drawings or specs: - Exposed surfaces: - Wood species. - Cut. - Grain direction. - Matching of veneer leaves. - Veneer matching within the panel face. - Semi-exposed surfaces: Melamine (thermoset decorative panels). - Finishes are shop applied: - Transparent Finish: Pre-catalyzed lacquer. - Sheen: Flat, satin, semi-gloss, gloss. - Transparent finish is either clear or stained to add color. - Cores are MDF: - Basis of Design Product: MDF by Rosebug, formaldehyde-free. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 064116 - PLASTIC-LAMINATE-FACED ARCHITECTURAL CABINETS |
|  | - Grade is AWI Custom. - Construction: Frameless. - Door and Drawer Front: Flush overlay. - Exposed Surfaces: - Exposed surfaces: Plastic Laminate - grades are indicated in the specs. - Semi-exposed surfaces: Melamine (thermoset decorative panels). - Cores are MDF: - Basis of Design Product: MDF by Rosebug, formaldehyde-free. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 064216 - FLUSH WOOD PANELING |
|  | - This section specifies veneer-faced wood paneling, with transparent finish, stained or clear. - Size and type of samples are applicable to the Project. - If mockups are required, description is in the specs, location is in the Drawings. - Paneling Rules: If there is other woodwork in the project with the same veneer as the paneling, the woodwork veneer in the other locations is described as “Match Paneling”. - Grade is AWI Premium. - Joints: Reveal size or describe inserts. - Frames. - Wood Veneer - Include the following in the specs or the drawings: - Wood species. - Cut. - Grain direction. - Matching of veneer leaves. - Veneer matching within the panel face. - Finishes are shop applied: - Transparent Finish: Pre-catalyzed lacquer. - Sheen: Flat, satin, semi-gloss, gloss. - Transparent finish is either clear or stained to add color. - Cores are MDF: - Basis of Design Product: MDF by Rosebug, formaldehyde-free. - Installation is by Z clips (preferable) or wood hanging strips. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 066411 - GLASS-FIBER REINFORCED PLASTIC (FRP) PANELING |
|  | - This section specifies wall panels at utilitarian spaces like janitor closets. - Basis of Design Manufacturer: Marlite FRP Wall Panels. - Include color and type of panels in the Drawings. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 064600 - wood trim |
|  | - This section specifies wood wall base and miscellaneous trim, including frames. - If wood base is at wood-veneer-paneling, core is MDF and AWI Grade and veneer match paneling. - NOTE: Solid wood looks different from veneer - cuts are not the same, so it does not look the same. - If wood base is not at wood paneling: - Veneered Trim: Core is MDF and veneer is your choice. - AWI Grade Custom or Premium. - Transparent Finish: Pre-catalyzed lacquer. - Solid Wood Trim: Indicate species, transparent finish as above. - AWI Grade Custom or Premium. - Opaque Finish: MDF, painted. - AWI Grade: Custom. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |

DIVISION 06 - WOOD.PLASTICS, AND COMPOSITES

## END OF TECH-CHECKLIST

## FROM THE AWI Standards - Arrangement of veneers