The content of the Tech-Check Lists is based on our specs an

tech-check specs architectural  I Division 08

This is a template, so some items will apply to your project, and others won’t or will be missing.

When using the list, make the changes needed for your project.

d MASTERSPEC®.

| ## ◻ | ## 081113 - HOLLOW METAL DOORS AND FRAMES |  |
| --- | --- | --- |
|  | - This section specifies steel doors and frames, rated and non-rated, non-insulated, interior and exterior door frames and doors, galvanized for exteriors and shop primed for interiors, to be field painted. - Basis of Design Manufacturer: Assa Abloy (CECO or Curries) - Section includes fire-rated doors when required by code. - Borrowed-lites are included in the section if in the project. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |  |
| ## ◻ | ## 081216 - ALUMINUM FRAMES |  |
|  | - This section specifies interior clear anodized doors and side-lights fames. - Basis of Design Manufacturer: Raco (only for one panel, no insulating glass) - Section includes fire-rated frames (up to 1 hour) when required by code. - Borrowed-lites are included in the section if in the project. - LEED PRements and Submittals included at the beginning of the section. |  |
| ## ◻ | ## 081416 - FLUSH WOOD DOORS |  |
|  | - This section specifies flush wood doors with opaque finish (paint), veneered faces, or plastic laminate faces. - Basis of Design Manufacturer: Eggers or Marshfield (Masonite) - Cores are MEF for non-rated doors and particleboard or mineral core for rated. - Painted doors are AWI Custom Grade with MDF faces. - Finish Type is specified. - Wood veneered doors are AWI Custom Grade except for posh projects, specify Premium Grade. - Veneer species, cut and placement of veneer next to each other and within the face f the doors is specified. - Finish is specified. - If veneered door is within a wood panel room or wall, this is indicated in the specs. - Acoustics performance requirements are specified if needed. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |  |
| ## ◻ | ## 083113 - ACCESS DOORS AND FRAMES |  |
|  | - This section specifies access panels and frames, steel to be field painted - Basis of Design Manufacturers typical units: Larsen’s or JL Industries. - Basis of Design Manufacturers for gypsum board ceilings at public areas: Intexforms. - Location of panels is not know until construction. - Rated requirements and products are included unless there are no rated assemblies in the project. - Locks are specified if required. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |  |
| ## ◻ | ## 083216 - SLIDING ALUMINUM-FRAMED GLASS DOORS |  |
|  | - This section specifies sliding aluminum-framed doors for exterior locations. - Basis of Design Manufacturers: CRL, Kawneer; Oldcastle. https://www.kawneer.com/kawneer/north_america/en/product.asp?cat_id=1343&prod_id=1846&desc=sliding-track-mall-storefront-door-panel - Performance requirements are included. - Glazing is included (these are glazed by the manufacturer). - Hardware is included in the section or it is in the Drawings. - Finish is included. - Details match product selected. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |  |
| ## ◻ | ## 083323 - OVERHEAD COILING DOORS |  |
|  | - This section specifies overhead coiling doors manually or motor operated, fire rated or not, for exteriors and interiors, shop finished. - Basis of Design Manufacturer: Overhead Doors (Cookson). https://www.overheaddoor.com/rolling-steel-service-doors - If possible, require single source responsibility for coiling and sectional doors in the project. - Performance requirements are included. - If motor operated, coordination with electrical work requirement is included. - If rated, fire rating requirements are included. - Curtain door of specified model is described. - Insulated is specified at exterior doors with at occupied interior spaces. - Locking devices are included and hopefully coordinated with Owner. - Shop finish is specified and color selected (or “selected from manufacturer’ standard colors” added). Custom colors are expensive. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |  |
| ## ◻ | ## 083613 - SECTIONAL DOORS |  |
|  | - This section specifies manually or motor operated sectional doors (there are not fire-rated sectional doors), steel only. - Basis of Design Manufacturer: Overhead Doors (Cookson). https://www.overheaddoor.com/rolling-steel-service-doors - If possible, require single source responsibility for coiling and sectional doors in the project. - Performance requirements are included. - If motor operated, coordination with electrical work requirement is included. - Curtain door of model selected is described. - Insulated is specified at exterior doors with at occupied interior spaces. - If vision panels are required they are included in the spec. - Locking devices are included and hopefully coordinated with Owner. - Shop finish is specified and color selected (or “selected from manufacturer’ standard colors” added). Custom colors are expensive. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |  |
| ## ◻ | ## 084113 - ALUMINUM-FRAMED ENTRANCES AND STOREFRONTS |  |
|  | - This section specifies storefront framing and manual-swing entrances. - Basis of Design Manufacturers: Kawneer and Oldcastle. https://www.kawneer.com/kawneer/north_america/en/product_category.asp?cat_id=1342&desc=fixed-aluminum-framing-storefront-glazing-systems - If possible, require single source responsibility for all the glazed assemblies in the building envelope. - Energy performance requirements are specified. - Structural and other performance requirements are included. - Only systems with thermal performance systems are OK for exterior applications. - If calculations are needed, these are usually by the fabricator. If “delegated design” is required, include the structural parameters in the Delegated Design Section. - Finish type is included - color for coatings and type of anodized (clear, black, etc.) - Check samples required by the section to make sure that is what you want. - Check that warranties required are included. - Check that details storefront, entrances, and installation match the section. - Entrance door is specified with the right rating (medium use, use type, hardware, finish) - Type of glass and glass position (center, back, front) in the frame is specified. - Field testing is included. - Butt joints (if any): Silicone sealant is specified. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |  |
| ## ◻ | ## 084116 - ALL-GLASS ENTRANCES AND STOREFRONTS |  |
|  | - This section specifies storefront framing and manual-swing entrances. - Basis of Design Manufacturers: Kawneer and Oldcastle. https://www.kawneer.com/kawneer/north_america/en/product_category.asp?cat_id=1342&desc=fixed-aluminum-framing-storefront-glazing-systems - If possible, require single source responsibility for all the glazed assemblies in the building envelope. - Energy performance requirements are specified. - Structural and other performance requirements are included. - Only systems with thermal performance systems are OK for exterior applications. - If calculations are needed, these are usually by the fabricator. If “delegated design” is required, include the structural parameters in the Delegated Design Section. - Finish type is included - color for coatings and type of anodized (clear, black, etc.) - Check samples required by the section to make sure that is what you want. - Check that warranties required are included. - Check that details storefront, entrances, and installation match the section. - Entrance door is specified with the right rating (medium use, use type, hardware, finish) - Type of glass and glass position (center, back, front) in the frame is specified. - Field testing is included. - Butt joints (if any): Silicone sealant is specified. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |  |
| ## ◻ | ## 084213 - ALUMINUM-FRAMED ENTRANCES |  |
|  | - This section specifies aluminum-framed manual-swing entrances for exteriors. - Basis of Design Manufacturers: US Aluminum, Kawneer and Oldcastle. https://www.kawneer.com/kawneer/north_america/en/product_category.asp?cat_id=1339&desc=commercial-aluminum-swing-door-entrance-systems - If possible, require single source responsibility for all the glazed assemblies in the building envelope. - Energy performance requirements are specified. - Traffic requirement is included (Moderate or High) - Finish type is included - color for coatings and type of anodized (clear, black, etc.) - Check that warranties required are included. - Check that details match the section. - Hardware is specified. - Type of stile (medium or Wide only, no Narrow because hardware does not fit) - Type of glass and glass position (center, back, front) in the frame is specified. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |  |
| ## ◻ | ## 084226 - ALL-GLASS ENTRANCES |  |
|  | - This section specifies frameless manual-swing entrances for exteriors. - Basis of Design Manufacturers: CRL (these are Blumcraft), Kawneer and Oldcastle. http://www.crl-arch.com/product_page/doors_entrances/glass_entrance_doors.html - If possible, require single source responsibility for all the glazed assemblies in the building envelope. - Metal components finish is included. - Check that warranties required are included. - Check that details match the section. - Hardware is specified (comes with the door). - Type of glass is specified (comes with the door). - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |  |
| ## ◻ | ## 084413 - glazed-aluminum curtain walls |  |
|  | - This section specifies aluminum framed traditional or unitized curtain wall systems. - Basis of Design Manufacturers: Kawneer and Oldcastle. https://www.kawneer.com/kawneer/north_america/en/product.asp?cat_id=1992&prod_id=1805&desc=thermal-break-ssg-aluminum-curtain-walls - If possible, require single source responsibility for all the glazed assemblies in the building envelope. - Energy performance requirements are specified. - Structural and other performance requirements are included. - Only systems with thermal performance systems are OK for exterior applications. - If calculations are needed, these are usually by the fabricator. If “delegated design” is required, include the structural parameters in the Delegated Design Section. - Finish type is included - color for coatings and type of anodized (clear, black, etc.) - Check samples required by the section to make sure that is what you want. - Check that warranties required are included. - Check that the section matches the curtain wall details curtain and installation. - Type of glass and glass position (center, back, front) in the frame is specified. - Field testing is included. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |  |
| ## ◻ | ## 084423 - structural-sealant-glazed curtain walls |  |
|  | - This section specifies curtain wall with butt joints (vertical, horizontal, or both ways) traditional or unitized curtain wall systems. - Basis of Design Manufacturers: Kawneer and Oldcastle. https://www.kawneer.com/kawneer/north_america/en/product_filter.asp?cat_id=1992 - If possible, require single source responsibility for all the glazed assemblies in the building envelope. - Energy performance requirements are specified. - Structural and other performance requirements are included. - Only systems with thermal performance systems are OK for exterior applications. - If calculations are needed, these are usually by the fabricator. If “delegated design” is required, include the structural parameters in the Delegated Design Section. - Finish type is included - color for coatings and type of anodized (clear, black, etc.) - Check samples required by the section to make sure that is what you want. - Check that warranties required are included. - Check that the section matches the curtain wall details curtain and installation. - Type of glass is specified. - Field testing is included. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |  |
| ## ◻ | ## 084523 - fiberglass-sandwich-panel assemblies |  |
|  | - This section specifies panels for walls, roof, skylights, and canopies. - Basis of Design Manufacturers: Kalwall. https://www.kalwall.com/kalwall-products/ - If possible, require single source responsibility for all the sandwich assemblies in the building envelope. - Energy performance requirements are specified. - Structural and other performance requirements are included. - If calculations are needed, these are usually by the fabricator. If “delegated design” is required, include the structural parameters in the Delegated Design Section. - Check that warranties required are included. - Check that the section matches the details. - Thickness, grid, both faces and aluminum framing are specified. - Finishes are included. - Field testing is included. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |  |
| ## ◻ | ## 086200 - unit skylights |  |
|  | - This section specifies plastic unit skylights (domes typically) with INTEGRAL CURBS. - Basis of Design Manufacturers: Bristolite, http://www.bristolite.com/interfaces/productsummary.aspx?M=2&V=1 - Require single source responsibility for all unit skylights in the project. - Energy performance requirements are specified. - Structural and other performance requirements are included. - Check that warranties required are included. - Check that the section matches the details. - Curb, framing, and plastic type are specified. - Unit size and shape, and finishes are included. - Structural curbs and safety screens are required. - Field testing is included. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |  |
| ## ◻ | ## 086300 - metal-framed skylights |  |
|  | - This section specifies plastic unit skylights (domes typically) with INTEGRAL CURBS. - Basis of Design Manufacturers: Bristolite, http://www.bristolite.com/interfaces/productsummary.aspx?M=2&V=1 - Require single source responsibility for all metal framed skylights in the project. - Energy performance requirements are specified. - Structural and other performance requirements are included. - If calculations are needed, these are usually by the fabricator. If “delegated design” is required, include the structural parameters in the Delegated Design Section. - Check that warranties required are included. - Check that the section matches the details. - Curb, framing, and glass type are specified. - Unit size and shape, and finishes are included. - Structural curbs and safety screens are required. - Field testing is included. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |  |
| ## ◻ | ## 087100 - door hardware |  |
|  | - This section specifies mechanical door hardware for swing doors. - Coordination with electrical work requirement is included. - Submittal of hardware schedule prepared under the supervision of the Installer Architectural Hardware Consultant requirement is included. - Warranties are included. - Performance requirements for fire rated doors, smoke control doors, electrified doors, means of egress doors and accessibility are included. - The hardware schedule and hardware groups are in the Drawings. |  |
| ## ◻ | ## 088000 - glazing |  |
|  | - This section specifies glass in the building envelope. - Basis of Design Fabricator: Viracon (does not manufacture glass; buys glass from the glass manufacturers and treats it (tempers, cuts, laminates, etc.) and then manufactures the glazing units (insulated glass, etc.) http://www.viracon.com/ - Warranties for each type of glass in the project are included. - Energy performance requirements are specified. - Structural and other performance requirements are included. - If calculations are needed, these are usually by the fabricator. If “delegated design” is required, include the structural parameters in the Delegated Design Section. - Check that all the glass types in the project are included in the spec. - Check that there is a schedule at the end of the section. - Sealants are included. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |  |
| ## ◻ | ## 088113 - decorative glass glazing |  |
|  | - This section specifies decorative glass in the building envelope. - Warranties for each type of glass in the project are included. - Energy performance requirements are specified. - Structural and other performance requirements are included. - If calculations are needed, these are usually by the fabricator. If “delegated design” is required, include the structural parameters in the Delegated Design Section. - Check that all the glass types in the project are included in the spec. - Check that there is a schedule at the end of the section. - Sealants are included. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |  |
| ## ◻ | ## 088300 - mirrors |  |
|  | - This section specifies wall mounted mirrors. - Warranties for each type of mirror in the project are included. - For backing of mirrors without lead, see manufacturer’s information. - If calculations are needed, these are usually by the fabricator. If “delegated design” is required, include the structural parameters in the Delegated Design Section. - Avoid attaching the mirror only with adhesive; specify mechanical attachments or a combination of mechanical attachments and adhesive. - Type of safety glazing is specified. - Trim or edge finish is included. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |  |
| ## ◻ | ## 088813 - fire-resistant glazing |  |
|  | - This section specifies fire-protection (not fire-resistant) glass and framing assemblies. - Basis of Design Fabricator: Safti-First, fire rating glass and framing. http://safti.com/ - Warranties for each type of glass in the project are included.. - Fire protection requirements are indicated. - Energy performance requirements are specified. - Structural and other performance requirements are included. - If calculations are needed, these are usually by the fabricator. If “delegated design” is required, include the structural parameters in the Delegated Design Section. - Check that all the glass types in the project are included in the spec. - Check that there is a schedule at the end of the section. - Sealants are included. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |  |
| ## ◻ | ## 089119 - fixed louvers |  |
|  | - Coordinate with mechanical consultant for airflow requirements, if needed for the project. - This section specifies extruded aluminum and formed metal louvers. - Basis of Design Fabricator: Construction Specialties. https://www.c-sgroup.com/architectural-louvers - Single source responsibility for all louvers in the project. - Field measurements requirement is included. - Finish is included. - Type of louver in the drawings matches the spec. - Blank-off panels are included if used. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |  |

DIVISION 08 - OPENINGS

## END OF TECH-CHECK LIST