The content of the Tech-Check Lists is based on our specs an

tech-check specs architectural  I Division 09

This is a template, so some items will apply to your project, and others won’t or will be missing.

When using the list, make the changes needed for your project.

d MASTERSPEC®.

DIVISION 09 - FINISHES

| ## ◻ | ## 033940 - CONCRETE SEALER-DENSIFIER |
| --- | --- |
|  | - This section specifies concrete sealers for utilitarian spaces where no other finish for the concrete slab is specified. - Basis of Design Product: SEAL HARD by L & M Construction Chemicals, Inc., 14851 Calhoun Road, Omaha, NE 68152 (800-362-3331) - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 092116.23 - GYPSUM BOARD SHAFT WALL ASSEMBLIES |
|  | - Check the following: - The shaft wall details correspond to the fire ratings shown in the plans and required. - At shafwalls scheduled to receive tile, the corresponding details show tile backing panels and tile backing panels are included in the specs. - A details for the jambs at the elevator-hoistway-entrances is included in the set. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 092400 - CEMENT PLASTERING (stucco) |
|  | - This section specifies stucco over steel framed walls, and over concrete and masonry walls. - Basis of Design Manufacturer: La Habra. - If mockup is needed, check that the mockup requirements are in the section and the locations are indicated on the Drawings - At steel-framed walls check that the following is specified: Building paper, mesh, brown and scratch coats, and finish coat. - At concrete and masonry walls check that the following is specified: Mesh, brown and scratch coats and finish coats. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 092216 - NON-STRUCTURAL METAL FRAMING |
|  | - This section specifies non-structural steel framing (non-load bearing studs and their accessories, basically). - Basis of Design Manufacturers: Clark Dietrich and Cemco. - Interior studs are 30 mils thick (drywall studs) typically. - Testing and inspecting by manufacturer in large or critical applications is required. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 092900 - GYPSUM BOARD |
|  | - This section specifies interior gypsum board for ceilings and partitions, and exterior gypsum board for ceilings and soffis. - Check that all types of gypsum board in the project are specified in the section. - Interior: - Gypsum board for ceilings 1/2 thick. - Type X for partitions. - Mold-Resistant. - Tile backer units at showers and wet walls where tile is scheduled. - Exterior: Glass-Mat Gypsum Sheathing Board or Exterior Gypsum. - Finishing levels are indicated. - Acoustical sealant is included. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 093013 - CERAMIC TILING (STONE TILING SIMILAR) |
|  | - This section specifies ceramic tiling installation at partitions, walls, and floors. - Basis of Design Product for Installation Materials: Mapei. - Single source responsibility for installation materials. - Check samples to see if that is what you want. - If mock-ups are required, check that the description is in the section and the location is indicated on the Drawings. - Check that the following systems are scheduled at the end of the section for regular and large-format tiles as applicable to the project: - At concrete or masonry walls at dry areas. - At steel framed partitions over gypsum board at dry areas. - At steel framed partitions over tile backing panels at wet areas. - At floors over crack-isolation membrane at dry areas. - At floors over waterproofing membrane at wet areas. - If tile is too heavy, structural studs might be necessary to support the tile. If this is the case, then the structural engineer has to design the framing. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 095113 - ACOUSTICAL PANEL CEILINGS |
|  | - This section specifies ceiling acoustical panels and exposed suspension systems. - Single source responsibility for suspension systems and panels in each assembly is required. - Surface-Burning requirements are included. - Check samples to see if that is what you want. - If mock-ups are required, check that the description is in the section and the location is indicated on the Drawings. - If fire-rated systems are included, check that requirements are included. - Check that accessories in the specs are the same as in the details. - Tolerances are included. - Special inspection is included. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 096513 - RESILIENT BASE AND ACCESSORIES |
|  | - This section specifies rubber or vinyl wall base and flooring accessories. - Check samples to see if that is what you want. - Accessories types match the details. - Wall Base: - Height matches details. - Base is provided in manufacturer’s standard coil length. - Inside and outside corners are pre-fabricated. - Stairs: - Types of stair accessories match drawings. - Testing for moisture in stairs concrete fill is included. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 096516 - RESILIENT SHEET FLOORING |
|  | - This section specifies glue-down resilient vinyl or rubber flooring. - Check samples to see if that is what you want. - Field conditions (HVAC working) are included. - Fire test Response Characteristics are included. - Types of flooring included match project selections. - Concrete slab moisture testing is included. - Installation method is correct (heat-welded or chemically bonded seams). - Installation of integral curb is included. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 096519 - RESILIENT TILE FLOORING |
|  | - This section specifies glue-down resilient vinyl or rubber tile flooring. - Check samples to see if that is what you want. - Field conditions (HVAC working) are included. - Fire test Response Characteristics are included. - Types of flooring included match project selections. - Concrete slab moisture testing is included. - Installation method is correct (heat-welded or chemically bonded seams). - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 097200 - WALL COVERINGS |
|  | - This section specifies vinyl, textile, and paper coverings. - Verify that samples are what you want. - Mockups are included if required. Locations should be shown in the Drawings. - Level finish at gypsum partitions is GA5 or GA4 if fabric is thin and gypsum board joints can telegraph. - Lighting is installed and on. HVAC system is working. - Wall covering product complies with surface burning characteristics and fire-growth contribution requirements (this is your responsibility, not the Contractor’s). - Installation instructions match type of wall covering. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 097713 - STRETCHED-FABRIC WALL SYSTEMS |
|  | - This section specifies site-upholstered wall systems. - Basis of Design Manufacturer: Whisper Walls - Review samples to make sure you get what you want. - Mockups are included if required. Locations should be shown in the Drawings. - Installer is authorized by manufacturer. - Lighting is installed and on. HVAC system is working. - Core material is as need for the project (acoustical, tackable, impact-resistant, etc.) - If required, core is fire-retardant. - Joints between panels are indicated (butt or reveal). - Fabric installation description is as required. - Installation tolerances are indicated. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 099113 - EXTERIOR PAINTING |
|  | - This section specified exterior painting (paints are “makeup”; coatings provide additional protections.) - Basis of Design Manufacturer: Sherwin Williams - Paint colors scheduled on the Drawings indicate only the colors chosen which will be match by the manufacturer of the paint systems specified. - Review the paint schedule at the end of the section and make sure it has all the substrates scheduled for painting as well as the correct sheens. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 099123 - INTERIOR PAINTING |
|  | - This section specified interior painting (paints are “makeup”; coatings provide additional protections). - Basis of Design Manufacturer: Sherwin Williams. - Paint colors scheduled on the Drawings indicate only the colors chosen which will be match by the manufacturer of the paint systems specified. - Review the paint schedule at the end of the section and make sure it has all the substrates scheduled for painting as well as the correct sheens. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 099600 - HIGH-PERFORMANCE COATINGS |
|  | - This section specifies surface preparation and paints for exposed structural steel and other exposed miscellaneous steel, aluminum or other metals that need field painting. - Basis of Design Manufacturer: Rust-Oleum. - Colors scheduled on the Drawings indicate only the colors chosen which will be match by the manufacturer of the paint systems specified. - Review the paint schedule at the end of the section and make sure it has all the substrates scheduled for painting as well as the correct sheens. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section.A joint cover schedule is added at the end of the section. Tags are the same as in the Drawings. |
| ## ◻ | ## SECTION 099653 - ELASTOMERIC COATINGS |
|  | - This section specifies surface preparation and paints for concrete and masonry substrates. - Basis of Design Manufacturer: Sherwin Williams. - Colors scheduled on the Drawings indicate only the colors chosen which will be match by the manufacturer of the paint systems specified. - Review the paint schedule at the end of the section and make sure it has all the substrates scheduled for painting as well as the correct sheens. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 099726 - CEMENTITIOUS COATINGS |
|  | - This section specifies surface preparation and paints for concrete and masonry substrates. - Basis of Design Manufacturers: Sherwin Williams and Tex-Cote. - Colors scheduled on the Drawings indicate only the colors chosen which will be match by the manufacturer of the paint systems specified. - Review the paint schedule at the end of the section and make sure it has all the substrates scheduled for painting as well as the correct sheens. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |

## END OF TECH-CHECKLIST