The content of the Tech-Check Lists is based on our specs and MASTERS

tech-check specs architectural  I Division 13

This is a template, so some items will apply to your project, and others won’t or will be missing.

When using the list, make the changes needed for your project.

PEC®.

DIVISION 13 - SPECIAL CONSTRUCTION

| ## ◻ | ## 133419 - METAL BUILDING SYSTEMS |
| --- | --- |
|  | - This section specifies PRE-FABRICATED industrial metal buildings. - If the metal building manufacturer is under a separate contract, then its work is “Owner provided - like furniture or equipment - the fabricator’s work is DESIGN-BUILD and we do not design, only coordinate as far as the building interfaces with our design, and we do not provide the data for the calculations or this section. - If the metal building manufacturer is a subcontractor under this Contract then the design is DELEGATED DESIGN, and our structural engineer provides the data for the calculations. - The manufacturer designs and calculates the building. Our structural engineer provides the data for the calculations. - It is VERY IMPORTANT that there is a close coordination between the Contractor and us through the design stage so that there are no surprises at the end of the process. - Ask the contractor to supply progress drawings as the design process advances to make sure that the metal building design is coordinated with ours. - Before the design process begins, ask through the Contractor if there are restrictions or preferences in the design, components, size, etc. of the prefabricated building so you are aware of them when you are working in your portion of the design. - Ensure that connections to utilities are coordinated. - Field measurements requirement is included. - Coordination paragraphs are included and modified for the project - ask G. - Pre-installation paragraph is included. - Samples required are correct. - Manufacturer and erector qualifications paragraph is included. - Warranties are included. - If possible, run this section through our structural engineer for review. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |
| ## ◻ | ## 134900 - RADIATION PROTECTION |
|  | - This section in RMW projects specifies only lead-lined gypsum, lead glass, and lead doors. Anything more than this should be handled by a consultant or Owner-Provided package. - Pre-installation meeting requirements are included. - Field measurements requirement is included. - Products are installed by fabricator - no exceptions. - Description of products matches products selected. - Review drawing notes and make sure they are correct. - Installation instructions match details. - LEED PROJECTS: Requirements and Submittals included at the beginning of the section. |

END OF TECH-CHECKING LIST